LED stage lighting equipment manual
Sky curtain four-eye two-color audience light-reflector cup
[image: image1.png]



Please read the manual carefully before use
Compared with LED moving head lights and traditional par lights, LED moving head lights have more rich color effects, richer color mixing effects, and the three primary colors of LED color mixing effects have risen to 16700000. LED moving head light categories: divided into two categories according to usage conditions, one, commercial lighting, also known as floodlights. Mainly plays the role of lighting, high-power LED lights, LED lights are usually used as a single-color small-volume lamp and are used in large shopping malls, hotels, guesthouses, fountain squares, etc. Second, stage lights, mainly used for lighting, color change, and creating stage atmosphere. LED lights have red, green, blue, and white colors, and are often used in stage performances, wedding ceremonies, exhibitions, and conference venues.

   1.Safety instructions           

Please read the manual carefully to understand the use, operation, and maintenance information of this product.
Please keep the manual for future review. If you sell it to other users, please make sure they have received this manual as well.

All equipment comes from the manufacturer in good condition. Please operate according to the instructions. Damage caused by failure to comply with the instructions is not covered by the warranty.

After unpacking, please check whether the product has been damaged during transportation before use.

This product is for indoor use only and can only be used in a dry environment.

It is recommended that a technician familiar with this product operate this product.

Please use a safety rope during installation. Please do not lift the entire product by the lamp head.

This product must be installed in a ventilated environment and at least 50cm above the ground.

Please ensure that there are no blocked vents, causing the product to overheat during operation.

Before operation, please ensure that the voltage and power frequency meet the product requirements.

The yellow and green conductors must be grounded to avoid electric shock.

The maximum temperature is TA: 40℃. Please stop operating when the ambient temperature exceeds the maximum temperature.

Please do not connect this product to other dimmers.

When used for the first time, there may be smoke or an unpleasant smell, which will disappear after a few minutes of operation.

Before use, make sure there are no flammable objects around to avoid a fire!

Please check the power cord carefully and replace it immediately if there is any damage.

Avoid long-term operation to avoid shortening its service life. Avoid any flammable liquids, water, or metal objects entering the lamp.

If an accident occurs, please cut off the power supply immediately.

Do not use it in a dirty or dusty environment, and remember to clean the device frequently.

Please prevent children from touching or operating the device.

Do not touch any wires during operation to avoid the risk of electric shock.

Please avoid entanglement between the power cord and other cables.

Do not connect the main power supply when replacing fuses, bulbs or repairs.

When replacing fuses, only use the same type of fuse.

Stop operation immediately in case of serious operating failure.       

     Installation

    This product needs to be installed through the screw holes on the bracket. You must always ensure that this product is firmly installed to avoid vibration or sliding of the device during operation. Always

Make sure that the bracket on which the product is installed is safe and can support the weight of the product 10 times. At the same time, when installing the device, you must always use a safety rope that can support the weight of the product 12 times.

一、Key operation and digital tube display instructions
[image: image2.jpg]585888
© ©® © O

MENU UP DOWN ENTER




MENU：function key
 UP：plus key   DOWN：minus key   ENTER：confirmation key  
Digital tube display menu instructions
	Serial number
	Menu
	Value
	Function description

	1
	A001
	001 - 512
	DMX512 address code setting

	2
	6CH
	6/12
	Channel mode setting

	3
	U255
	000 - 255
	White dimming, from dark to bright

	3
	C255
	000 - 255
	Warm color dimming, from dark to bright

	5
	F000
	005 - 255
	Total strobe, from slow to fast

	6
	CL01
	01 - 25
	Built-in color

	7
	EF01
	01 - 30
	Self-propelled effect, press the OK key to set the speed SP01 - SP15

	8
	SU01
	01 - 02
	Voice control effect, press the OK key to set the sensitivity SE01 - SE15


二、DMX channel function table
  6-channel mode (basic function)
	Channel
	Value
	Function description

	CH1
	000-255
	Total dimming, from dark to bright

	CH2
	005-255
	Total strobe, from slow to fast

	CH3
	000-255
	U dimming, from dark to bright (effect color)

	CH4
	000-255
	C dimming, from dark to bright (effect color)

	CH5
	000-009
	No function

	
	010-019
	Static color

	
	020-029
	Breathing effect

	
	030-039
	Jumping effect

	
	040-049
	Gradual effect

	
	050-059
	Pulse effect

	
	060-066
	Effect-1

	
	067-073
	Effect-2

	
	074-080
	Effect-3

	
	...
	...

	
	...
	...

	
	229-235
	Effect-24

	
	236-242
	Effect-25

	
	243-249
	Effect-26

	
	250-252
	Voice control mode 1

	
	253-255
	Voice control mode 2

	CH6
	000-255
	Static color mode: color selection

Effect mode: adjust speed, from slow to fast

Voice control mode: adjust voice control sensitivity, from low to high (the highest sensitivity is the sensitivity set in the menu)


12-channel mode (single head control)
	Channel
	Value
	Function description

	CH1
	000-255
	Total dimming, from dark to bright

	CH2
	005-255
	Total strobe, from slow to fast

	CH3
	000-255
	U1 dimming, from dark to bright

	CH4
	000-255
	C1 dimming, from dark to bright

	CH5
	000-255
	U2 dimming, from dark to bright

	CH6
	000-255
	C2 dimming, from dark to bright

	CH7
	000-255
	U3 dimming, from dark to bright

	CH8
	000-255
	C3 dimming, from dark to bright

	CH9
	000-255
	U4 dimming, from dark to bright

	CH10
	000-255
	C4 dimming, from dark to bright

	CH11
	000-009
	No function

	
	010-019
	Static color

	
	020-029
	Breathing effect

	
	030-039
	Jumping effect

	
	040-049
	Gradual effect

	
	050-059
	Pulse effect

	
	060-066
	Effect-1

	
	067-073
	Effect-2

	
	074-080
	Effect-3

	
	...
	...

	
	...
	...

	
	229-235
	Effect-24

	
	236-242
	Effect-25

	
	243-249
	Effect-26

	
	250-252
	Voice control mode 1

	
	253-255
	Voice control mode 2

	CH12
	000-255
	Static color mode: color selection

Effect mode: adjust speed from slow to fast

Voice control mode: adjust voice control sensitivity from low to high (the highest sensitivity is the sensitivity set in the menu)


四、Technical parameters:

1. Input voltage: AC100-240V/ 50-60ZH

2. Input power: 250W

3. Light source parameters: 4*50W dual-color COB lamp beads;

4. Light source life: 50,000 hours

5. Control mode: DMX512, voice control, self-propelled, master-slave control;

6. Control interface: 3-core XLR head (XLR-3P)

7. Channel mode: 6/12 channels

8. Strobe speed: 0-20 times/second (speed adjustable)

9. Protection level: IP20

10. Dimensions: 350*350*115mm

11. Product net weight: 4.7Kg
