炫彩图案

1、工作电压: AC95V-245V/50Hz

2、产品功率:100W (图案12W，线条24W， 光束24W，50MW红绿激光）

3、图案参数:4个图案，4个线条，8个光束，2个红激光，2个绿激光

4、灯珠参数：20颗单色灯珠，4颗50mW红绿激光

5、国际通用DMX512，22通道

6、工作模DMX512，主/从机，自走，声控

7、调光方式:0”100%超平滑调光

8、频闪: 20次/秒

| 22 通道 | 功能 | DMX数值 | 描述 |
| --- | --- | --- | --- |
| 1 | X轴电机 | 0-255 | 0~255 0-540度旋转 |
| 2 | Y轴电机 | 0-255 | 0~255 0-180度旋转 |
| 3 | Y轴电机 | 0-255 | 0~9(不转)10~127正转由慢到快，128~255反转由快到慢 |
| 4 | XY电机速度 | 0-255 | 0~255由快到慢 |
| 5 | 总调光 | 0-255 | 从暗到亮进行线性调光 |
| 6 | 频闪 | 0-9 | 无功能 |
|  |  | 10-255 | 慢到快 |
| 7 | 红色图案 | 0-255 | 红色调光从暗到亮 |
| 8 | 绿色图案 | 0-255 | 绿色调光从暗到亮 |
| 9 | 蓝色图案 | 0-255 | 蓝色调光从暗到亮 |
| 10 | 白色图案 | 0-255 | 白色调光从暗到亮 |
| 11 | 红色线条 | 0-255 | 红色调光从暗到亮 |
| 12 | 绿色线条 | 0-255 | 绿色调光从暗到亮 |
| 13 | 蓝色线条 | 0-255 | 蓝色调光从暗到亮 |
| 14 | 白色线条 | 0-255 | 白色调光从暗到亮 |
| 15 | 红色光束 | 0-255 | 红色调光从暗到亮 |
| 16 | 黄色光束 | 0-255 | 黄色调光从暗到亮 |
| 17 | 粉色光束 | 0-255 | 粉色调光从暗到亮 |
| 18 | 无功能 | 0-255 | NF |
| 19 | 红激光 | 0-255 | 红激光调光从暗到亮 |
| 20 | 绿激光 | 0-255 | 绿激光调光从暗到亮 |
| 21 | 宏功能控制 | 0-50 | 通道1到通道20有效 |
|  |  | 51-150 | 随机自走 |
|  |  | 150-255 | 声控自走 |
| 22 | 复位 | 0-250 | NONE |
|  |  | 251-255 | 3秒复位 |

A001-A512 地址设置

A 1-A 9 自走模式   末尾数字设置速度快慢

S1-1-S1-9 声控模式 末尾数字设置声控灵敏度

- Colorful pattern

- Operating voltage: AC95V-245V/50Hz

- Product power: 100W (pattern 12W, line 24W, beam 24W, 50MW red green laser)

- Pattern parameters: 4 patterns, 4 lines, 8 beams, 2 red lasers, 2 green lasers

- Lamp bead parameters: 20 monochromatic lamp beads, 4 50MW red green laser

- Control mode: DMX512, 22 channels for international use

- Working mode: DMX512, master/slave, self-walking, voice control

- dimming mode: 0~100% ultra-smooth dimming

- Stroboscopic: 20 times per second

| 22 channel | function | DMX value | Describe |
| --- | --- | --- | --- |
| 1 | X Motor | 0-255 | 0~~255 0-540 degree rotation |
| 2 | Y Motor | 0-255 | 0~~255 0-180 degree rotation |
| 3 | Y-axis motor non-polar rotation | 0-255 | 0~9 (no rotation) 10~127 forward rotation from slow to fast, 128~255 reverse rotation from fast to slow |
| 4 | XY speed | 0-255 | 0~255 from fast to slow |
| 5 | dimmer | 0-255 | Linear dimming from dark to light |
| 6 | Strobe | 0-9 | None |
|  |  | 10-255 | form slow to fast |
| 7 | Red pattern | 0-255 | Red Dimming from Dark to Light |
| 8 | Green pattern | 0-255 | Green Dimming from Dark to Light |
| 9 | Blue pattern | 0-255 | Blue Dimming from Dark to Light |
| 10 | White pattern | 0-255 | White Dimming from Dark to Light |
| 11 | Red line | 0-255 | Red Dimming from Dark to Light |
| 12 | Green line | 0-255 | Green Dimming from Dark to Light |
| 13 | Blue line | 0-255 | Blue Dimming from Dark to Light |
| 14 | White line | 0-255 | White Dimming from Dark to Light |
| 15 | Red beam | 0-255 | Red Dimming from Dark to Light |
| 16 | Yellow beam | 0-255 | Green Dimming from Dark to Light |
| 17 | Pink beam | 0-255 | Blue Dimming from Dark to Light |
| 18 | NF | 0-255 | NF |
| 19 | red laser | 0-255 | red laser Dimming from Dark to Light |
| 20 | green laser | 0-255 | green laser Dimming from Dark to Light |
| 21 | Macro function | 0-50 | Channels 1 to 20 are valid |
|  |  | 51-150 | auto run |
|  |  | 150-255 | voice autorun |
| 22 | reset | 0-250 | none |
|  |  | 251-255 | reset after 3 second |

- A001-A512 address settings

- A1-1-A1-9 self mode at the end of the digital setting speed

- S1-1-S1-9 Set the voice sensitivity at the end of the voice mode